Research Report

Migration from IPv4

to IPv6

Module Code: COMP40046

Module Name: Networks and Hardware

Student ID: 24030180

Contents

The Specification	1

IP addressing	3

Basics of IPv4	3

Basics of IPv6	3

Advantages of Migrating from IPv4 to IPv6 for BIC	5

Address Space	5

Security	5

Future Proofing	5

Performance and Efficiency	5

Disadvantages of Migrating from IPv4 to IPv6	6

Address Space	6

Security	6

Future Proofing	6

Performance and Efficiency	6

Migrating from IPv4 to IPv6	8

Methods	8

Dual-Stacking	8

Tunnelling	8

NAT-PT	9

Costs	9

Cost and Timeframe of Training Staff	9

Cost of Installing and Upgrading Infrastructure	10

Timeframe of Installing and Upgrading Infrastructure	10

Conclusion	12

Should BIC Fully Adopt IPv6?	12

How Should BIC Migrate to IPv6?	12

References	13

Word Count with Tables - 3484

Word Count without Tables - 3285

Introduction

## The Specification

This report is intended to discuss the advantages and disadvantages of Belfast Insurance Company (hereafter referred to as BIC) fully adopting IPv6 across their offices and the possible options for a future migration from IPv4 to IPv6 across all offices.

Although IPv6 is overall a superior technology to IPv4 (as will be made apparent further in this report) and its eventual adoption is inevitable (barring interruption by a better technology), there are drawbacks to its adoption. Therefore, this report will also ascertain whether it makes sense for BIC to implement a full migration to IPv6.

A simulation has been created alongside this report, designed to show what the BIC network would look like before a possible IPv6 migration. As such, this report will assume the design in the simulation has been implemented and will use that as a starting point.

Currently, there are three offices within the BIC network: London, Dublin and the Belfast HQ forming a Wide Area Network (WAN). In the simulation created along with this report, the Belfast HQ has both IPv4 and IPv6 addressing on its BADMIN router, and five insurance record servers. This report assumes that the simulation is accurate and all the networks before the upgrade are exactly as seen in the simulation (except where the simulation abstracts away details, such as where groups of hosts are represented by a single host).

All other networked devices across the offices only have IPv4 addressing. Although a breakdown could be made on an office-by-office basis, as this report concerns fully adopting IPv6 across all offices, the numbers of devices needing upgrading to IPv6 amounts to:

- 5 Cisco 2911 routers

- 6 Cisco 3650-24PS switches

- 598 hosts, probably representing PCs and laptops for employees, with unknown operating systems

There are currently six subnets present on the simulation, with this Variable Length Subnet Mask (VLSM) Table showing the breakdown of the BIC Network for IPv4 addresses (note that for the Hosts Available, each router on each subnet would by convention (Cisco Networking Academy, 2013) occupy the last IP address on the private network, reducing the Hosts Available by one in reality):

Table 1 - BIC VLSM Table

| Subnet Name | Hosts Needed | Hosts Available | Unused Hosts | Network Address | Office Location | Router Public Interfaces IP addresses |
| --- | --- | --- | --- | --- | --- | --- |
| Administration (BADMIN) – Belfast HQ | 200 (includes 5 insurance record servers with dual stack IPv4- IPv6 already) | 254 | 54 | 192.168.0.0 /24 | Belfast HQ | 145.45.5.1, 146.45.5.1, 147.45.5.1, 148.45.5.1, 149.45.5.1, 150.45.5.1 (connects to all other routers on WAN) |
| Vehicle Insurance Department (VID) - London | 176 | 254 | 78 | 192.168.1.0 /24 | London | 150.45.5.2 |
| Life Insurance Department (LID) - London | 124 | 126 | 2 | 192.168.2.0 /25 | London | 149.45.5.2 |
| Home Insurance Department (HID) - Dublin | 59 | 62 | 3 | 192.168.2.128 /26 | Dublin | 147.45.5.2 |
| Travel Insurance Department (TID) - Dublin | 26 | 30 | 4 | 192.168.2.192 /27 | Dublin | 148.45.5.2 |
| Management Department (BMAN) – Belfast HQ | 18 | 30 | 12 | 192.168.2.224 /27 | Belfast HQ | 146.45.5.2 |

Each subnet has its own router, with each subnet’s router connecting through a single hop to the BADMIN router, which is dual-stacked and connects to the ISP’s router.

It is assumed for the purpose of this report that fully adopting IPv6 means having all devices on the BIC network capable of IPv6 interfaces and capable of IPv6 communication and does not mean the deprecation of IPv4 capability.

# IP addressing

IP addresses are numeric identifiers assigned to each machine on a network (Lammle & Buhaglar, 2024) which allows for devices to communicate with each other by routing data packets.

## Basics of IPv4

An IPv4 address is made up of 32 bits, allowing for a up to 4,294, 967, 296 possible IP addresses. An example of this would be the IP address 192.0.2.53, with each decimal number representing an ‘octet’ (eight bits or a byte) (IANA, 2025).

Public-facing IPv4 addresses must be unique in the world, and the Internet Assigned Numbers Authority (IANA) issues blocks of IPv4 addresses to regional registrars, who issue them to Internet Service Providers (ISPs), who then distribute them to the public (Davies, 2019).

IPv4 addressing was originally split between classes ranging from A through to E, with some special IP addresses reserved elsewhere (such as address 127.0.0.1 being used to troubleshoot network issues with the ping command (Meridian Outpost, 2025)). Only classes A-C were used for general addressing (Agapidis, 2025). Here is a table showing the ranges of IP addresses for classes A-C, along with the subnet mask, which divides up the network portion of the address from the host portion:

Table 2 - IPv4 Classful Addressing

| Class | Range | Subnet Mask |
| --- | --- | --- |
| A | 0.0.0.0 - 127.255.255.255 | 255.0.0.0 |
| B | 128.0.0.0 - 191.255.255.255 | 255.255.0.0 |
| C | 192.0.0.0 - 223.255.255.255 | 255.255.255.0 |

Because classful IPv4 addressing is wasteful (a Class A network can have up to 16,777,214 hosts (Odom, 2024)), VLSM (Variable Length Subnet Mask) or CIDR (Classless Inter-Domain Routing) was invented to divide networks up more granularly via variable subnets, allowing for better allocation.

IPv4 addressing is split between public and private IPv4 addresses. This is because there are more devices in use than the 4,294, 967, 296 available addresses, so each device cannot be wholly unique. NAT (Network Address Translation) is used to convert between public and private IP addresses (as otherwise multiple devices could have the same IP address on different networks, making routing impossible).

## Basics of IPv6

An IPv6 address is made up of 128 bits, allowing for 2128 possible IP addresses (Wikipedia, 2025). IPv6 addresses are represented by eight groups of 16 bits in hexadecimal format that are colon-delimited, an example being 2001:0db8:85a3:0000:0000:8a2e:0370:7334. Because of the large number of addresses, it is possible to assign every single device on the internet a unique IP address, removing the need for NAT that IPv4 relies on to slow the dwindling number of public addresses.

IPv6 also has a simplified header structure (headers allows routers to process packets of data), with only 7 fields, compared to IPv4’s 13 (optionally 14). This design ‘allows routers to process packets faster and thus improves throughput and delay.’ (Wetherall & Tanenbaum, 2010).

IPv6 also has inbuilt IPsec (Internet Protocol Security), which provides secure IP communications by ‘authenticating the sender and providing integrity protection plus, optionally, confidentiality for the transmitted data.’ (Abernathy & Hayes, 2024).

# Advantages of Migrating from IPv4 to IPv6 for BIC

## Address Space

The larger address size would allow BIC to ignore constraints on potential future hosts, as currently the private IPv4 address range being used on the internal network is 192.168.0.0/16, which can contain up to 65,534 usable addresses whereas for private IPv6 networks, the Unique Local Address (similar to IPv4 private addresses) range fd00::/8 is used (Hagen, 2014), which can contain up to 2120  addresses, an almost unlimited amount that ensures a ‘high probability of uniqueness’ (Graziani, 2017). The biggest advantage a larger address size for private networks would give BIC is that, for departments such LID, HID and TID, where the number of available hosts is running out (see Table 1), usage of Unique Local Address subnets would result in much larger numbers of hosts available.

## Security

The improved security through IPsec helps secure communications against outside IPv4 systems and the fact that IPv6 encrypts traffic at all layers (Wills, 2022) would be beneficial for BIC’s network security regarding BIC’s handling and processing of personal data. Whilst the 5 insurance servers on the BADMIN network already have dual-stacked IPv4-IPv6 configurations, having end-to-end encryption (Beasley & Nilkaew, 2021) and protection against tampering through IPsec on hosts would reduce the possibility of data leaks and being in violation of General Data Protection Regulation (GDPR). Fines from GDPR can be either £10 million or 2% of global turnover for minor infringements or £20 million or 4% of global turnover for more serious infringements (GDPRAdvisor, 2025) so migrating to IPv6 may be worth it for security alone depending on BIC’s financial status.

## Future Proofing

Migrating to IPv6 will also future proof BIC’s network. Available IPv4 addresses have already run out, as Réseaux IP Européens Network Coordination Centre (RIPE NCC) allocated the last of their IPv4 addresses in 2019 (RIPE NCC, 2025). At some point in the future IPv4 will be completely obsolete and decommissioned, although there is no predetermined date for this, Scott Hogg, an IPv6 consultant, wrote in a blog: ‘Predicting that IPv4 will “go the way of the dodo” sometime after 2030 would be a reasonable estimate.’ (Hogg, 2018). This means migrating to IPv6 now could prevent a hasty and ill-conceived migration later.

## Performance and Efficiency

The improved routing efficiency from IPv6’s simplified header structure ‘means that QoS (Quality of Service) is maintained even across the public internet, which is impossible with IPv4’ (Gilchrist, 2016). This would allow BIC to have some future customer facing internet presence from within their internal network, allowing BIC’s network to handle growth in the business without responsiveness seeming ‘slow’ to customers. It would also improve speeds on BIC’s internal network.

# Disadvantages of Migrating from IPv4 to IPv6

## Address Space

Although migrating to IPv6 would allow for a larger address space, it might be easier and cheaper for BIC to simply expand its subnets with limited space by resizing ‘existing subnets to provide more addresses and to expand the subnet size to a larger CIDR block’ (Montgomery, 2023). The combined costs of compatibility issues, having to upgrade hardware and operating systems, retraining network engineers and potential downtime due to mistakes and unforeseen issues may make the Return on Investment not worth it compared to something which would be relatively routine for experienced network engineers.

## Security

Security wise, although many features of IPv6 make it more secure, there are some details that need to be taken into consideration. Due to its longer existence, IPv4 protocols ‘benefited from the work of security researchers for much longer, and thus IPv4 implementations are generally more robust than their IPv6 counterparts.’ (Internet Society, 2025). Whilst the larger address base of IPv6 makes it almost impossible to brute force a network scan (Santos, 2007), this also means there are more source addresses for attackers to use, making IP filtering and block lists less effective (Chapple, et al., 2024) and they might use more sophisticated reconnaissance methods to find BIC’s networks. Chapple, et al. also state: ‘A second issue is that secure deployment of IPv6 requires that all security filtering and monitoring products be upgraded to fully support IPv6 prior to enabling the protocol on the production network. Otherwise, IPv6 will serve as a covert channel, as it will be unmonitored and unfiltered.’, meaning that if not done correctly by engineers experienced with IPv6, security could be heavily compromised.

## Future Proofing

Another factor to consider is that migrating from IPv4 to IPv6 may not be something that needs to be considered currently by BIC. There may be more pressing business issues and as already stated, there is no official predetermined date for IPv4 decommissioning.

## Performance and Efficiency

The performance gains of IPv6 may go unnoticed by customers to a website if BIC were to add web servers to their internal network as discussed in the Advantages section. The following graphic (Figure 1) (Think with Google and Pingdom, 2024) shows that customer retention only starts to be significantly affected after 3 seconds of page load speed. So, if the IPv4 network is sufficient to make sure that page loads are generally under 3 seconds, then migrating to IPv6 may not make much of a difference to customers.

Figure 1 - Page Load Speed Chart

It is also unlikely, due to the relatively low number of hosts on the internal network, that performance speeds would be noticed by employees.

# Migrating from IPv4 to IPv6

## Methods

There are three common methods for migrating from IPv4 to IPv6: Dual-Stacking, Tunnelling and Network Address Translation-Protocol Translation (NAT-PT).

### Dual-Stacking

Dual-Stacking is the most common form of migrating from IPv4 to IPv6 because it’s the ‘easiest’ (Lammle & Buhaglar, 2024). It involves allowing devices to use both the IPv4 protocol and IPv6 protocol depending on which recipient device they are communicating with, with Domain Name Systems (DNS) determining which stack to use over the internet, if ‘a dual-stacked device queries the name of a destination and DNS gives it an IPv4 address (a DNS A Record), it sends IPv4 packets. If DNS responds with an IPv6 address (a DNS AAAA Record), it sends IPv6 packets.’ (Juniper Networks, 2023). The router and the five insurance servers on the BADMIN subnet on the simulation are already dual-stacked, meaning this method of migration has already been partially implemented at BIC.

### Tunnelling

Tunnelling is another way of migrating from IPv4 to IPv6. There are many different types of tunnelling, such as host-to-host, host-to-router, and router-to-router, as shown in the image below (Graziani, 2017)

Figure 2 - Types of Tunnelling

It is mostly used to allow devices in IPv6 ‘islands’ to communicate to other IPv6 devices over an IPv4 network by encapsulating IPv6 packets into IPv4 packets at a tunnel entrance and then decapsulating them at the tunnel exit (catchpoint, 2025). For BIC’s six subnets, this would involve using the dual-stacked routers as tunnel entry points and exit points. It would also mean that communication within the subnets could continue as hosts and switches are given IPv6 capability as they would simply communicate on IPv4 until upgraded.

### NAT-PT

NAT-PT is a method of migrating from IPv4 to IPv6 and is ‘a type of NAT that translates IPv6 addresses into IPv4 addresses and vice versa.’  and ‘the aim of NAT-PT is to use translation only when there is no other method to allow IPv6-only nodes to communicate with IPv4-only domains.’ (Desmeules, 2003). Despite being available as a last-resort, NAT-PT has been deprecated, with the Internet Engineering Task Force (2007) stating:

This document draws the conclusion that the technical and operational

difficulties resulting from these issues, especially the possible

future constraints on the development of IPv6 networks (see

Section 5), make it undesirable to recommend NAT-PT as described in

[RFC2766] as a general purpose transition mechanism for

intercommunication between IPv6 networks and IPv4 networks.

As such, it is reasonable to conclude that NAT-PT would not be a suitable candidate for migration, because of the possibility of Dual-Stacking or Tunnelling, and any further exploration would be fruitless.

## Costs

### Cost and Timeframe of Training Staff

If BIC needs to train network administrators or engineers in IPv6 implementation, then it should supply them with training and qualifications before implementing IPv6. Cisco, who are one of the ‘world’s leading information technology and networking companies’ (Nickolas, 2022) offers the ‘CCNA, CCNP, or CCIE Routing and Switching certification exams - certified by the IPv6 Education Certification Program - Cisco also offers additional training with the “IPv6 Fundamentals, Design and Deployment Course (IP6FD)" which has been Gold certified by the IPv6 Forum.’ (Cisco Admin, 2020). Information on the cost of training for these qualifications is represented in the table below:

Table 3 - CISCO Qualifications

| Qualification | Cost per Person | Timeframe for Qualification |
| --- | --- | --- |
| CCNA | $300 (Cisco, 2025) | Week-long course, or self-paced (Cisco Learning Locator, 2025) |
| CCNP | $400 (Cisco, 2025) | Week-long course, or self-paced (Cisco Learning Locator, 2025) |
| CCIE | $400 (Cisco, 2025) | Week-long course, or self-paced (Cisco Learning Locator, 2025) |
| IP6FD | $900 (The Cisco Learning Network Store, 2025) | Self-paced (180 days of access) (The Cisco Learning Network Store, 2025) |

This table suggests that if every applicable employee took every course available, it should not take more than half a year for all training to have taken place if scheduled properly.

### Cost of Installing and Upgrading Infrastructure

For Dual-Stacking, the BADMIN subnet is already dual-stacked on its insurance servers and its router. The Cisco 2911 routers used on all the other subnets have IPv6 capabilities (Cisco, 2017), as do the 3650-24PS switches used on all subnets (Cisco, 2018). This means that the main cost of installing and upgrading on the BIC network would be on the man-hours required to upgrade and the cost of upgrading end devices if they ran on operating systems that don’t support IPv6. It is highly unlikely that any of the operating systems of the hosts would not support IPv6 as ‘By 2011, all major operating systems had production-quality IPv6 implementations.’ (Wikipedia Contributors, 2025), with initial IPv6 support for Linux being implemented in 1996 (OpenNET, 2025); Windows in 2001 with Windows XP (although trial versions were available for Windows 95 and 98) (Pallas, 2022); Mac in 2002 with the OS X 10.2 Jaguar release (van Beunum, 2012).

For Tunnelling, tunnels would have to be set up between the BADMIN HQ router and the other IPv4 routers, which would need IPv6 capable hosts and switches to take in the traffic once past the tunnel. Tunnelling ‘requires management and this is costly and complex.’ (UK Gov Department for Business Innovation and Skills, 2010) and the configuration and upkeep would probably be the largest factor standing out compared to the Dual-Stack solution.

### Timeframe of Installing and Upgrading Infrastructure

Although exact timings for the installing and upgrading of infrastructure cannot be given, both Dual-Stacking and Tunnelling would be very similar, as a lot of the infrastructure is in place. However, Tunnelling would take time to setup, configure and maintain, with encapsulation and decapsulation having to be done, and would still require the setting up of dual-stacked or IPv6-only devices in the subnets. A tunnel from the BADMIN router to the ISP could be worth it, as a solution for enabling IPv6-only devices on the WAN to communicate with the internet through the tunnel (Hagen, 2014). Given that there are 609 devices currently needing IPv6 capability on the WAN, even upgrading one device a day in a Dual-Stacking approach would allow the physical migration to be done within two years.

# Conclusion

## Should BIC Fully Adopt IPv6?

It is likely that at some point in the next two decades, BIC will need to upgrade to IPv6, as it is very likely that IPv4 will be deprecated (Hogg, 2018). However, it is not necessary for this to done hastily, as BIC has room to re-subnet on their private IPv4 network, and this would allow for significant accommodation of new hosts. There is also room on the private network for other subnets, however acquiring public IP addresses for new routers on these subnets may prove difficult (Taylor, 2021).

Migrating to IPv6 would also be beneficial for reasons of protecting BIC’s client data. It may also allow for more capability in terms of web servers and would make communication over the internal network faster. When weighing the benefits and cons, it is clear BIC should fully adopt IPv6.

## How Should BIC Migrate to IPv6?

BIC should upgrade its entire network over the course of several years, like so:

- BIC should either train existing network engineers or administrators that are unfamiliar with IPv6, using the Cisco qualifications. They should allow between 6-8 months for employees to upskill, allotting time during the week where they can train alongside their usual duties.

- Any hosts or host operating systems which are incompatible with IPv6 should be upgraded. In the worst-case scenario of all hosts and operating systems being incompatible, upgrading the 598 hosts could take from six months to a year. This could be done at the same time as the upskilling of employees, however.

- Upgrade the BADMIN subnet to full dual-stack capability, making sure to pilot test it and make it secure as a proof of concept. Allow six months for full subnet deployment.

- From there, keep upgrading the next largest subnet to dual-stack capability in line with how the BADMIN subnet was upgraded, until all subnets on the WAN are dual-stacked. Because there would be more experience at this point, allow up to two years for full deployment.

- It is likely that the ISP router will also be dual-stack by this point (as ISPs continue to adopt IPv6) (ipv6 test, 2025), but if it is still IPv4 only, a tunnelling solution may be required from the BADMIN router to the ISP network to any IPv6 only networks on the internet. This could take some time to configure and get right, so if required this could take up to six months.

- At some point in the future, drop IPv4 from devices when IPv4 becomes redundant (no time scale, as this relies on external factors).

# References

Abernathy, R. & Hayes, D. R., 2024. CISSP Cert Guide. 5th ed. s.l.:Pearson IT Certification.

Agapidis, L., 2025. Classful vs Classless IP Addressing - Comparison and Differences. [Online] 
Available at: https://www.networkstraining.com/classful-vs-classless-ip-addressing/
[Accessed 15th January 2025].

Antoniou, T., 2024. The Impact of Website Performance on Your Business. [Online] 
Available at: https://pressidium.com/blog/websites-performance-impact-on-your-business/
[Accessed 17th January 2025].

Beasley, J. S. & Nilkaew, P., 2021. Networking Essentials: A CompTIA Network+ N10 - 008 Textbook: Chapter 11. 6th ed. s.l.:Pearson IT Certification.

catchpoint, 2025. IPv6 Tunnelling. [Online] 
Available at: https://www.catchpoint.com/benefits-of-ipv6/ipv6-tunnelling
[Accessed 18th January 2025].

Chapple, M., Stewart, J. M. & Gibson, D., 2024. ISC2 CISSP Certified Information Systems Security Professional Official Study Guide. 10th ed. s.l.:Sybex.

Cisco Admin, 2020. Getting on Board with IPv6. [Online] 
Available at: https://learningnetwork.cisco.com/s/article/getting-on-board-with-ipv6#:~:text=By%20taking%20the%20CCNA%2C%20CCNP%2C%20or%20CCIE%20Routing,be%20able%20to%20demonstrate%20IPv6%20knowledge%20and%20skills.
[Accessed 18th January 2025].

Cisco Learning Locator, 2025. Implementing and Administering Cisco Solutions 2.1. [Online] 
Available at: https://learninglocator.cloudapps.cisco.com/#/home
[Accessed 18th January 2025].

Cisco Networking Academy, 2013. Network Basics Companion Guide: Chapter 8. s.l.:Cisco Press.

Cisco, 2017. Cisco 2900 Series Integrated Services Routers Data Sheet. [Online] 
Available at: https://www.cisco.com/c/en/us/products/collateral/routers/2900-series-integrated-services-routers-isr/data_sheet_c78_553896.html
[Accessed 18th January 2025].

Cisco, 2018. Cisco Catalyst 3650 Series Switches Data Sheet. [Online] 
Available at: https://www.cisco.com/c/en/us/products/collateral/switches/catalyst-3650-series-switches/data_sheet-c78-729449.html
[Accessed 18th January 2025].

Cisco, 2025. Cisco Certified Network Associate. [Online] 
Available at: https://www.cisco.com/site/us/en/learn/training-certifications/exams/ccna.html#tabs-35d568e0ff-item-194f491212-tab
[Accessed 18th January 2025].

Cisco, 2025. Implementing and Operating Cisco Collaboration Core Technologies. [Online] 
Available at: https://www.cisco.com/site/us/en/learn/training-certifications/exams/clcor.html#tabs-35d568e0ff-item-194f491212-tab
[Accessed 18th January 2025].

Davies, G., 2019. Need for Unique IP Addresses. In: Networking Fundamentals. s.l.:Packt Publishing.

Desmeules, R., 2003. Cisco Self-Study: Implementing IPv6 Networks (IPV6). s.l.:Cisco Press.

GDPRAdvisor, 2025. GDPR Fines and Penalties: What You Need to Know to Avoid Costly Mistakes. [Online] 
Available at: https://www.gdpradvisor.co.uk/gdpr-fines-and-penalties#:~:text=The%20GDPR%20establishes%20two%20tiers%20of%20administrative%20fines,whichever%20is%20higher%20%E2%80%93%20For%20more%20serious%20infringements.
[Accessed 17th January 2025].

Gilchrist, A., 2016. Industry 4.0: The Industrial Internet of Things. s.l.:Apress.

Graziani, R., 2017. IPv6 Fundamentals: A Straightforward Approach to Understanding IPv6. 2nd ed. s.l.:Cisco Press.

Hagen, S., 2014. IPv6 Essentials. 3rd ed. s.l.:O'Reilly Media, Inc.

Hogg, S., 2018. When Will We Stop Using IPv4?. [Online] 
Available at: https://blogs.infoblox.com/ipv6-coe/when-will-we-stop-using-ipv4/#:~:text=There%20is%20no%20specific%20predetermined%20date%20when%20the,will%20be%20no%20%E2%80%9Cflag%20day%E2%80%9D%20for%20IPv4%20decommissioning.
[Accessed 17th January 2025].

IANA, 2025. Number Resources. [Online] 
Available at: https://www.iana.org/numbers
[Accessed 15th January 2025].

Internet Engineering Task Force, 2007. RFC 4966. [Online] 
Available at: https://www.ietf.org/rfc/rfc4966.txt
[Accessed 18th January 2025].

Internet Society, 2025. IPv6 Frequently Asked Questions (FAQ). [Online] 
Available at: https://www.internetsociety.org/deploy360/ipv6/security/faq/
[Accessed 17th January 2025].

ipv6 test, 2025. IPv6 in United Kingdom. [Online] 
Available at: https://ipv6-test.com/stats/country/GB
[Accessed 24th January 2025].

Juniper Networks, 2023. Understanding Dual Stacking of IPv4 and IPv6 Unicast Addresses. [Online] 
Available at: https://www.juniper.net/documentation/us/en/software/junos/is-is/topics/concept/ipv6-dual-stack-understanding.html
[Accessed 18th January 2025].

Lammle, T. & Buhaglar, J., 2024. CompTia Network+ Study Guide, Chapter 7. 6th ed. s.l.:Sybex.

Meridian Outpost, 2025. The Five IPv4 Classes - Quick Reference. [Online] 
Available at: https://www.meridianoutpost.com/resources/articles/IP-classes.php
[Accessed 15th January 2025].

Montgomery, T., 2023. AWS Certified Advanced Networking Study Guide. 2nd ed. s.l.:Sybex.

Nickolas, S., 2022. Top 5 Companies Owned by Cisco. [Online] 
Available at: https://www.investopedia.com/articles/company-insights/090716/top-5-companies-owned-cisco-csco.asp
[Accessed 18th January 2025].

Odom, W., 2024. CCNA 200-301 Official Cert Guide, Volume 1. 2nd ed. s.l.:Cisco Press.

OpenNET, 2025. Linux IPv6 HOWTO: Chapter 2: Basics. [Online] 
Available at: https://www.opennet.ru/docs/HOWTO/Linux+IPv6-HOWTO/basic-history-ipv6-linux.html
[Accessed 18th January 2025].

Pallas, V. D., 2022. IPv6 on Windows | The Definitive Guide. [Online] 
Available at: https://pall.as/ipv6-on-windows/#:~:text=Microsoft%20was%20surprisingly%20quick%20after%20the%20first%20IPv6,Windows%20XP%20broad%20support%20for%20IPv6%20was%20integrated.
[Accessed 18th January 2025].

RIPE NCC, 2025. RIPE NCC IPv4 Pool. [Online] 
Available at: https://www.ripe.net/manage-ips-and-asns/ipv4/ipv4-pool/
[Accessed 17th January 2025].

Santos, O., 2007. End-to-End Network Security: Defense-in-Depth: Chapter 11. s.l.:Cisco Press.

Taylor, A., 2021. Overcoming the Challenges of IPv4 Exhaustion. [Online] 
Available at: https://www.enterprisetimes.co.uk/2021/10/20/overcoming-the-challenges-of-ipv4-exhaustion/
[Accessed 24th January 2025].

The Cisco Learning Network Store, 2025. IPv6 Fundamentals, Design, and Deployment (IP6FD) v4.1. [Online] 
Available at: https://learningnetworkstore.cisco.com/on-demand-e-learning/ipv6-fundamentals%2C-design%2C-and-deployment-ip6fd-v4.1/CSCU-LP-IP6FD-V4-028389.html
[Accessed 18th January 2025].

Think with Google and Pingdom, 2024. Page Load Speed vs Bounce Rate. s.l.:s.n.

UK Gov Department for Business Innovation and Skills, 2010. IPV6 ROLLOUT IN THE UK. [Online] 
Available at: https://assets.publishing.service.gov.uk/government/uploads/system/uploads/attachment_data/file/78236/10-1229-ipv6-rollout-in-the-uk.pdf
[Accessed 18th January 2025].

van Beunum, I., 2012. The future is forever: the state of IPv6 in the Apple world. [Online] 
Available at: https://arstechnica.com/gadgets/2012/05/the-future-is-forever-the-state-of-ipv6-in-the-apple-world/#:~:text=IPv6%20has%2079%2C228%2C162%2C514%2C264%2C337%2C593%2C543%2C950%2C336%20times%20more%20addresses%20than%20the,OS%20X%2010.2%20Jaguar%20release%20in
[Accessed 18th January 2025].

Wetherall, D. J. & Tanenbaum, A. S., 2010. Computer Networks. Fifth ed. s.l.:Pearson.

Wikipedia Contributors, 2025. IPv6 Deployment. [Online] 
Available at: https://en.wikipedia.org/wiki/IPv6_deployment
[Accessed 18th January 2025].

Wikipedia, 2025. IPv6 Address. [Online] 
Available at: https://en.wikipedia.org/wiki/IPv6_address#Address_space
[Accessed 15th January 2025].

Wills, M., 2022. (ISC)2 SSCP Systems Security Certified Practitioner Official Study Guide, Chapter 6. 3rd ed. s.l.:Sybex.